Electric two-wheel motor, controller and battery information:

Controller:

- 48v, 1000w

- Power cutoff: (45v-54v)

- MOSFET:

- In three phase bridge configurations, (3phase*2 MOSFET per phase= 6 MOSFET)

- Microcontroller:

-Texas instrument (TI) C2000 (TMS320F28069)

- HALL SENSOR:

- No. of Hall sensor: 3

-Placement: 120 electrical degrees apart

-Purpose: Provide rotor position feedback for commutation.

- Current Sensors

- Voltage Regulators

- Capacitors and Inductors

BLDC MOTOR:

- Brushless dc motor

- Power supply: 45v-54v

- No. of phase: 3

- Rated rpm: 550

- Rated torque: 30Nm

- Maximum amp consumed without load in full throttle: 2.5amp constant

- Maximum amp consumed with load in full throttle: 4.87amp constant

FIG: Experimental results of PWM output pulses of S1 and S6 with half throttle (hall state 100)

Fig: Experimental results of PWM output pulses of S1 and S6 with full throttle (hall state 100)

Fig : Results of speed of BLDC motor (rated load torque of 6Nm at 025 to 04sec (940 rpm) (From Research paper )

Battery:

- Type: Lithium-ion battery cell

- Battery pack of: 48v,25ah

- Each cell: 3.7v (3.4v-4.2v)

- Series Connections: 13 cells in series to achieve 48V

- Parallel Connections: 8 cells in parallel for each series string to achieve 25Ah

- Total Cells=13×8=104 cells

- BMS with 13 cell connection.

Thermal Analysis of Li-Ion Battery using Ansys CFD

Fig: Temperature distribution (°C) and velocity profile (m/s) at the end of discharge.

Device we used:

WIREING DIAGRAM